Chapter 1.11 - 1.15  Notes

- Lecture Notes

  - Amdahl’s Law

    - Deriving it starts from the expression

    - Overall speedup = execution time old / execution time new

    - Another way to visualize it is having multiple parallel tasks run on multiple processors for part of the execution time

      - With perfect partitioning and no communication overhead in the parallel portion

    - Its a law of diminishing returns

    - f is the program execution time

    - S is the speedup of a fraction f

  - ILLIAC IV

    - First massively parallel computer

  - The effort expended on achieving high parallel processing rates is wasted unless it is accompanied by achievements in sequential processing rates of very nearly the same magnitude.

  - Cray-1 supercomputer

    - Embodies amdahl’s wisdom.

    - Seymour cray improved the scalar performance as well as provided high performance register based vector units.

  - What is the overall speedup in program execution time if an enhancement with a speedup of 2 is available and can be used to enhance 40% of the execution time? You should use Amdahl’s Law and give your answer as a fraction.

    - Do not say that it is 25% faster than….

  - Consider a vector computer in which vector mode can provide a speedup of 10. Using Amdahl's Law, what amount of vectorization is required for an overall speedup of 5? Solve the problem algebraically and express the answer as a reduced fraction rather than a decimal or a percentage.

    - Overall speedup = 5, s = 10, find f

  - Older machines will be byte accessible.

  - Little endian addresses the least accessible byte

- Fallacies and Pitfalls 1.11

  - Pitfall

    - Expecting the improvement of one aspect of a computer to increase performance by an amount proportional to the size of the improvement.

  - Amdahl’s Law

    - A rule stating that the performance enhancement possible with a given improvement is limited by the amount that the improved feature is used.

    - It is a quantitative version of the law of diminishing returns.

    - We use it to estimate performance improvements when we know the time consumed for some function and its potential speedup.

    - Amdahl’s law, with the cpu performance equation, is a handy tool for evaluating potential enhancements.

    - Also used to argue for practical limits to the number of parallel processors.

    - For above, the multiply operations must run in 0 seconds to run the program 5 times faster.

    - 30 / 2 + (100 - 30)

      - 15 + 70

      - 85 seconds

    - If some aspect of a computer accounts for 50% of program execution time, what is the limit on how many times faster programs can run if engineers focus on improving that aspect?

      - 2 times

  - Fallacy: computer at low utilization use little power

    - Power efficiency matters at low utilizations because server workloads vary.

    - Even given five years to learn how to run the SPECpower benchmark well, the specially configured computer with the best results in 2020 still uses 33% of the peak power at 10% of the load.

    - If future servers used, say, 10% of peak power at 10% workload, we could reduce the electricity bill of datacenters and become good corporate citizens in an era of increasing concern about CO2 emissions.

  - Fallacy: Designing for performance and designing for energy efficiency are unrelated goals.

  - Pitfall: Using a subset of the performance equation as a performance metric.

    - One alternative to time is MIPS

    - MIPS

      - Million instructions per second

      - A measurement of program execution speed based on the number of millions of instructions.

      - MIPS is computed as the instruction count divided by the product of the execution time and 10^6

    - Three problems with MIPS

      - MIPS specified the instruction execution rate but does not take into account the capabilities of the instructions.

      - MIPS varies between programs on the same computer, thus a computer cannot have a single MIPS rating.

      - If a new program executes more instructions but each instruction is faster, MIPS can vary independently from performance.

- Concluding Remarks 1.12

  - Hardware and software designers construct computer systems in hierarchical layers, with each lower layer hiding details from the level above.

    - This is abstraction.

  - The most important example of abstraction is:

    - The interface between hardware and low-level software called the instruction set architecture.

  - Execution time is the only valid and unimpeachable measure of performance.

  - The key hardware technology for modern processors is silicon.

  - Two of the key ideas are exploiting parallelism in the program, typically today via multiple processors, and exploiting locality of accesses to a memory hierarchy, typically via caches.

  - Energy efficiency has replaced die area as the most critical resource of microprocessor design.

    - Conserving power while trying to increase performance has forced the hardware industry to switch to multi core microprocessors, thereby forcing the software industry to switch to programming parallel hardware.

    - Parallelism is now required for performance.

- Historical Perspective and Further Reading 1.13

  - J. Presper Eckert and John Mauchly at the Moore School of the University of Pennsylvania build the first operational electric general purpose computer called the ENIAC (electronic numerical integrator and calculator)

    - Funded by the United States Army

    - Operational during WWII, not publicly disclosed until 1946

    - Used for computing artillery firing tables

    - U-shaped

    - 80 feet long, 8.5 feet high, several feet wide

    - Each of the 20 10-digit registers was 2 feet long

    - Used 18,000 vacuum tubes

    - 8 orders of magnitude slower than modern machines, performing 1900 additions per second

    - Programming was done manually by plugging cables and setting switches, data was entered on punched cards.

      - For typical calculations, programming could take half an hour to a whole day.

    - Limited by a small amount of storage and tedious programming.

  - John von Neumann proposed a stored-program computer called EDVAC (Electronic Discrete Variable Automatic Computer)

    - Herman Goldstine distributed the memo but put von Neumann’s name on it.

    - Eckert and Mauchly worked on the machine but were not credited.

    - Eckert and Machly left the project in 1947 and this delayed completion until 1952.

  - Maurice Wilkes decided to build a stored-program computer named EDSAC (Electronic Delay Storage Automatic Calculator)

    - Became operational in 1949

    - World’s first full scale operational stored-program computer

      - A small prototype called the Mark-I (University of Manchester, 1948)  is the first operational stored-program machine

  - IAS machine was built by Julian Bigelow

    - Has a total of 1024 40-bit words, 10 times faster than ENIAC

    - Machine was inspired by the memo written by von Neumann memo

  - John Atanasoff

    - Built a machine in the 40s at Iowa State

    - Special purpose computer that wasn't completely operational

  - Konrad Zuse built a special purpose machine in late 30s/early 40s in Germany

    - Government didn’t fund it because of the war

  - Alan Turing built the Colossus in 1943 with a team at Bletchley Park

  - Howard Aiken built the Mark-I at Harvard

    - Followed this machine with a relay machine Mark-II, and a pair of vacuum tube machines, Mark-III and Mark-IV

    - Mark-III and Mark-IV had separate memories for instructions and data

    - Regarded as reactionary by advocates of stored program computers

  - Harvard Architecture

    - Coined to scribe machines with separate memories

    - Used today in a different sense, to describe machines with a single main memory but separate caches for instructions and data.

  - Whirlwind Project

    - Begun at MIT in 1947

    - Aimed at applications in real-time radar signal processing

    - Most important innovation was magnetic-core memory

    - Has 2048 16-bit words of magnetic core

    - Magnetic cores served as the main memory for 30 years

  - In December 1947, Eckert and Mauchly formed Eckert-Mauchly Computer Corporation.

    - Their first machine, the BINAC, was built for Northrop and was shown in August 1949

    - The firm was eventually acquired by Remington-Rand where they built the UNIVAC (Universal Automatic computer)

      - Designed to be sold as a general purpose computer

      - Correctly predicted the 1952 presidential election, withheld from broadcasting networks though.

      - First delivered in June 1951, UNIVAC I sold for about $1 Million and was the first successful commercial computer.

  - The IBM 701, first IBM computer, was shipped in 1952, 19 units were sold.

  - In 1964, after investing $5 Billion, IBM announced the System/360.

    - A family of computers developed to address a wide range of computing needs

    - Four models that varied in cost and performance by a factor of almost 10, it grows to a factor of 25 if we include models 20 and 30.

      - Model 40, 1.6 MHz, 32 KB–256 KB, $225,000

      - Model 50, 2.0MHz, 128KB–256KB, $550,000

      - Model 65, 5.0MHz, 256KB–1MB, $1,200,000

      - Model 75, 5.1MHz, 256KB–1MB, $1,900,000

  - In 1965, Digital Equipment Corporation (DEC) unveiled the PDP-8.

    - The world’s first minicomputer

    - Small machine that was under $20,000.

    - Minicomputers were the forerunners of microprocessors

      - Intel invited the first microprocessor in 1971, the Intel 4004

  - In 1963, the first supercomputer was announced

    - Extremely fast computers targeted to perform a large number of computations typically needed by scientific applications.

  - Seymour Cray led the design of the Control Data Corporation CDC 6600 in Minnesota and is the ‘father of supercomputing’.

    - Formed Cray Research INC in Wisconsin

    - In 1976, he announced the Cray-1.

      - The fastest in the world, the most expensive, and the computer with the best cost/performance for scientific programs.

      - Sales only a little over 80 supercomputers

  - Apple llc set standards for low cost, high volume, and high reliability.

    - Between 1977 and 1993, Apple produced about six million of these.

  - The IBM Personal Computer, announced in 1981, became the most popular microprocessor, with Microsoft being the most popular OS.

  - The first personal computers were quite simple, with little or no graphics capability, no pointing devices, and primitive operating systems compared to those of today.

  - Xerox Alto

    - Created as an experimental prototype of a future computer.

    - Inspired many architectural and software concepts that characterize modern desktops

    - Hundreds were made and many donated to universities

    - Technologies that it incorporated were:

      - A bit-mapped graphics display integrated with a computer

      - A mouse

      - A local area network (LAN) and was a precursor to the internet

      - A user interface based on windows featuring a what you see is what you get editor and interactive drawing programs

      - Both file servers and print servers were developed and interfaced via the local area network.

        - connections between LAN and wide are ARPAnet produced the versions of internet-stype networking

  - Original measure of performance was the time required to perform an individual operation, such as addition.

    - To consider differences like timing, etc, an instruction mis was calculated by measuring the relative frequency of instructions in a computer across many programs.

    - Multiplying the time each instruction took by its weight in the mix gave the user the average instruction time.

  - The Whetstone was a synthetic program created by measuring scientific programs written in Algol-60

    - Conveyed to Frotran and was widely used to characterize scientific program performance.

    - Typically quoted in Whetstones per second (number of executions of a single iteration of the Whetstone benchmark)

    - Dhrystone is another benchmark that is still used in some embedded computing circles.

  - Around the same time of Whetstone, kernel benchmarks became popular

    - Kernels are small, time-intensive pieces from real programs that are extracted and then used as benchmarks

    - Developed for benchmarking high-end computers, especially supercomputers

    - Livermore Loops and Linkpack are best examples.

      - The Livermore Loops consist of a series of 21 small loop fragments.

      - Linpack consists of a portion of a linear algebra subroutine package

    - Kernels often overstate the performance on real applications

    - Best used to isolate the performance of individual features of a computer and explain reasons for differences in the performance of real programs

  - An important advance in performance evaluation was the formation of the System Performance Evaluation Cooperative (SPEC) group in 1988.

    - SPEC comprises representatives of many computer companies—the founders being Apollo/Hewlett-Packard, DEC, MIPS, and Sun.

    - In 1991, a throughput measure was added, based on running multiple versions of the benchmark.

    - Other system benchmarks that include OS-intensive and I/O-intensive activities have also been added.

    - SPEC98 contained six floating-point benchmarks, but only four integer benchmarks

      - Calculating a single summary measurement using the geometric mean of execution times normalized to a VAX-11/780 meant that this measure favored computers with strong floating-point performance.

    - SPEC92 was introduced and incorporated additional benchmarks, dropped matrix300, and provided separate means for integer and floating-point programs (SPEC INT and SPECFP)

    - The SPECbase measure, which disallows program-specific optimization flags, was added to provide users with a performance measurement that would more closely match what they might experience on their own programs.

    - SPECFP showed the largest increase in numbers verses the base SPECFP measurement

    - The sole survivor from SPEC89 is the gcc compiler

  - Embedded processors have been around for a long time: The first minicomputers and first microprocessors were originally developed for controlling functions in a lab or industrial application.

    - Embedded processors grew in popularity due to their needs from video games to cell phones, etc.

    - The Embedded Microprocessor Benchmark Consortium (EEMBC) was created in 1997

      - A second generation of these benchmarks was announced in 2007.

    - In 2019, Embench was developed with the goal of replacing widespread use of synthetic programs like Dryhstone for embedded computing benchmarking.