Chapter 1.6 Notes

- Introduction 1.6

  - T = execution time

  - IC = instruction count

  - CPI = cycles per instruction

  - CCT = clock cycle time

  - CR = clock rate (note that CR = 1 / CCT)

  - T = IC * CPI * CCT or, T = IC * CPI / CR

  - Sanity check: just as in chemistry, physics, etc., do a dimensional analysis:

    - instructions * cycles/instructions * seconds/cycle => result will be in seconds

  - Speedup = old execution time / new execution time.

    - Note that the speedup is unitless.

    - Speedup = ( IC_old * CPI_old * CCT_old ) / ( IC_new * CPI_new * CCT_new )

    - See Lecture Notes 2 for examples

  - Workload = program and input

  - Configuration = CPU, cache, memory, I/O, OS, compiler, and optimizations

  - Elapsed time = program execution + I/O + wait, which is important to user

  - Execution time = user time + system time (but note that OS self measurement may be inaccurate)

  - CPU performance = user time on unloaded system, which is important to computer architect

  - Response time (latency) – time, between start and completion of a unit of work

  - Throughput (bandwidth) – rate, which is work units done per unit time, e.g., transactions/sec

    - Note that response time and throughput are reciprocals, i.e., throughput = 1 / response time

  - Marketing measures – the quest for single a marketing number usually leads to problems!

    - MFLOPS = millions floating point operations in program / (execution time * 106)

    - MIPS = millions of instructions per second count / (execution time * 106) = CR / (CPI * 106)

      - Note that MIPS as a performance measure is not comparable across different computer designs when the instructions between the designs vary significantly in complexity.

    - MIPS = Microprocessor without Interlocked Pipeline Stages (DIFFERENT THAN THE ONE ABOVE!)

    - RISC

    - CISC

  - As an individual computer user, we are interested in reducing response time.

    - Response time

      - The time between the start and completion of a task - also referred to as execution time.

      - The total time required for the computer to complete a task, including disk accesses, memory accesses, I/O activities, operating system overhead, CPU execution time, and so on.

  - Datacenter managers are often interested in increasing throughput or bandwidth.

    - Throughput

      - The total amount of work done in a given time. Also called bandwidth.

      - Another measure of performance, it is the number of tasks completed per unit time.

    - In most cases we need different performance metrics to benchmark personal mobile devices.

      - Mobile devices are more focused on response time

      - Servers are more focused on throughput.

  - For a given task, Computer X has less execution time (is faster) than Computer Y.

  - Desired is a measure of "performance" where bigger is better ("more performance" means faster).

  - One such performance measure is the inverse of execution time.

  - The performance of two computers can be related quantitatively. Computer X is n times as fast as computer Y.

  - The execution time of two computers can be related quantitatively. The execution time on computer Y is n times as long as computer X.

- Example:

  - If computer A runs a program in 10 seconds and computer B runs the same program in 15 seconds, how much faster is A than B?

    - We know that A is n times as fast as B if

    - Thus the performance ratio is 15/10 = 1.5

    - A is 1.5 times as fast as B.

    - We can also say that computer B is 1.5 times slower than computer A.

  - We usually say "improve performance" or "improve execution time" when we mean "increase performance" and "decrease execution time."

  - Performance = 1/ execution time

  - Time is the measure of computer performance:

    - The computer that performs the same amount of work in the least time is the fastest.

    - Program execution time is measured in sections per program.

    - The most straightforward definition of time is called wall clock time, response time, or elapsed time.

    - These all mean the total time to complete a task

  - We often want to distinguish between the elapsed time and the time over  which the processor is working on our behalf.

  - CPU Execution time/CPU Time recognizes this.

    - CPU time is the time the CPU spends computing for this task and does not include time spent waiting for I/O or running other programs.

      - The response time experienced by the user will be the elapsed time of the program, not the CPU time.

    - CPU can be further divided into User CPU Time (time spend in the program) and System CPU Time (time spent in the operating system performing tasks on behalf of the program)

    - We use ‘system performance’ to refer to elapsed time on unloaded system’ and ‘cpu performance’ to refer to user cpu time.

- 1.6.5 Work

  - Elapsed time is all time combined

    - 5+4+2+10 = 21 ms

  - User CPU Time is only user time

    - 5+10 = 15 ms

    - This is because the task runs for 5 ms then 10 ms. this does not count time waited or paused.

  - CPU time is the total time the cpu spends running or operating the task. So when it is not idle

    - 5+4+10 = 19 ms

  - The system performance is the elapsed time

    - 21 ms

  - CPU Performance is the user cpu time

    - 15 ms

- Almost all computers are constructed using a clock that determines when events take place in the hardware.

  - These discrete time intervals are called clock cycles.

    - Or ticks, clock ticks, clock periods, clocks, cycles.

  - The length of a clock period is referred to as both the time for a complete clock cycle (250 picoseconds/250 ps) and the clock rate (4 gigahertz/4 GHz)

- Clock cycle

  - Time for one clock period, usually of the processor clock which runs at a constant rate.

- Clock period

  - The length of each clock cycle.

- CPU execution time for a program = CPU clock cycles for a program X Clock cycle time

- CPU execution time for a program = CPU clock cycles for a program / clock rate

- Example

  - Our favorite program runs in 10 seconds on computer A, which has a 2 GHz clock. We are trying to help a computer designer build a computer, B, which will run this program in 6 seconds. The designer has determined that a substantial increase in the clock rate is possible, but this increase will affect the rest of the CPU design, causing computer B to require 1.2 times as many clock cycles as computer A for this program. What clock rate should we tell the designer to target?

    - First, find the number of clock cycles required for the program on A:

    - So CPU Clock Cycles A = 10 seconds X 2 X 10^9 cycles/second = 20 X 10^9 Cycles.

    - CPU Time B can be found using this:

    - To run a program in 6 seconds, B must have twice the clock rate of A.

- Execution time equals the number of instructions executed multiplied by the average time per instruction.

  - The number of clock cycles required for a program can be written as

  - CPU Clock Cycles = Instructions for a program X Average clock cycles per instruction

- CPI (Clock cycles per instruction)

  - An average of all the instructions executed in the program.

  - CPI Provides one way of comparing two different implementations of the same instruction set architecture.

  - Average number of clock cycles per instruction for a program or program fragment.

- ps stands for picoseconds

- ns stands for nanoseconds

- Instruction count

  - The number of instructions executed by the program.

- CPU time

  - Instruction count X CPI X Clock Cycle Time

  - Or (Instruction count X CPI) / Clock Rate

    - Clock rate is the inverse of clock cycle time

- Next will be the basic components of performance and how each is measured.

- Instruction count depends on the architecture, not on the exact implementation.

- The CPI depends on a variety of design details in the computer as well as a mix of instruction types executed.

- Today's processors can vary their clock rates, so we would need to use the average clock rate for a program.